Building an IBM Compatible

or How to be a real Clone

DJ Elliott

PC HELP

Box 19401

Baltimore, Md 21206

301-661-1921

GKHF68A-Prodigy

**********DISCLAIMER**********

Although I believe this to be a project that any reasonably intelligent

person can accomplish, the Author assumes no responsibility for failed

attempts, errors, omissions or injuries or losses connected to the use

of the information contained herein.  If you need help, get it.  If you

are not sure, ask.  So there.

Copyright 1990 DJ Elliott. All Rights Reserved.

If received via BBS

I INVITE YOUR COMMENTS AND YOUR CRITICISM.  PLEASE SEND ME  E-MAIL

WITH YOUR RESPONSE. YOU ARE FREE TO UPLOAD IT TO OTHER BBS'S IN

IT'S ENTIRETY WITH NO CHANGES TO THE TEXT. COMMENTS SHOULD BE

OUTSIDE OF THE ARTICLE.

DJE January,1991                                       Version 1.3

Revision History- Original November, 1989.

This is a living document. It is being corrected and expanded

constantly. Below is the date of the latest modification.

January 7, 1991

Yes, You Can!

Build your own IBM Compatible Computer

by DJ Elliott

With very little know how and using only what you already know about IBM

Compatible Computers, it is an easy and enjoyable task to assemble an

80286 Machine for a total of about $800.  This machine will look like a

store model and do everything a store bought system will do, and leave

you $1000 or more for other pursuits.

The Machine can be built in your spare time in a few days, or over a

number of weeks or months, adding the parts as you can afford them and

find them at the right price.

This author, over three months of occasional work and with no formal

training, built a 12 Mhz 80286 Compatible Machine with 2048K Memory, a

5 1/4 and 3 1/2 Inch drive, a 10 meg hard disk, Parallel and Serial

Ports, and a 1200 bps Modem. Actual work time was in the neighborhood of

10 hours. This unit is a virtual Rocket, running 700% faster than an IBM

XT (source: PC Tools Diagnostics).  A soldering gun was used ONCE, to

add a switch (optional for my uses).  The gun was the most technical

piece of equipment used.  There was no test equipment, special tools or

mathematics involved.  This article is written on the described Machine.

Since that time, I have assisted in the building of several other

machines, and advised on many others. Over 300 people have downloaded

the original article. Once you start, you are hooked. I have since

increased memory to 4 megs and bought a used 40 Meg hard drive at a show

for $40 and I constantly improve and upgrade the machine as I learn

more, which is the real lasting fun of the project.

Why build a computer instead of buying it? MONEY and fun.  This machine

would easily go for between $1200 and $3000. Most parts are under

warranty, and you are not "stuck" with a computer that does not work.

Just have the offending part replaced or serviced.

Compatibility-  What makes this process nearly idiot proof is the

architecture- everything plugs in or screws together.  If you can build

a model car with Legos or hook up a stereo, you can accomplish this.

IBM made the decision years ago that the MS-DOS PC would be an Open

Architecture System- a flash of brilliance that led to their domination

of the Market- and made Apple an also ran.

Whatever part it is that you are looking for, it is made by a number of

different manufacturers at a number of different price points, and,

wonder of wonders- they fit into the same slot the same way.  An ABC

Motherboard accepts a DEF Controller, which runs a GHI Hard Drive, which

fits into a JKL kit, and takes MNO disks.

Make your decision to build this project, don't look back, and plan on

your Check Writing, Tax Work, Recipes, Letter Writing, and work from the

office to be as close as your living room and as fast as your machine at

work.

This article has evolved from a six page general outline to almost book

length. As of this revision, the article has been split into two

distinct sections: a concise how-to of the basics, followed by lengthier

information that you will most definitely need wither to narrow down

your final buying decision or to support you as you debug your hardware.

The Basic Parts

There are a number of items you will need to get started and they are

easily obtained.  All compatibles have the following components:

*A Case

*A Motherboard (w/CPU and BIOS)

*A Power Supply

*A Keyboard (or other input device)

*A Monitor

*A Disk or Hard Drive (I/O device)

*ROM [Read Only Memory] -the basic instructions

*RAM [Random Access Memory]- your work space

*Controllers

The more useful options are:

A second Disk Drive or a Hard Drive;

a Modem;

a Mouse;

a Printer.

PREFACE

This article started at 5 pages. It is turning into a book. That's good

in that I'm passing along lots and lots of information that will save

you days or weeks of trouble and lots of dollars. It's bad in that the

technical jargon may turn you off. It shouldn't. The actual building

process is something you absolutely should be capable of doing. Here is

a one paragraph summary of what you will be doing:

You will buy a case, install the motherboard and set a few jumpers per

the documentation. You will fill some empty sockets with memory chips.

You will put in a little silver box that is your power supply. You now

have a computer. You will next insert a card into a slot and hook a

monitor to it. You will plug the keyboard into the back. You will plug

in a card which controls the disk drives, and hook the drives up. You

will turn on the machine and tell the computer about the stuff you have

added. That's it. Really. The balance of this article is to help you

decide which monitor, drives, etc. to buy, and then gives lots and lots

of advice for avoiding pitfalls along the way. Each of these pitfalls

caused me heartaches and headaches. I pass the solutions on to you so

that you may avoid the problems.

If you want to cut to the skinny, here is a short list of the parts for

a recommended system:

You may, for $500-700, buy the following in one morning at a computer

show-

A baby tower case with power supply, an 80286 motherboard, 2 meg of 1

Meg chips (18 chips), a 256K VGA Card, a Mono VGA monitor, a hard floppy

controller, a 5 1/4 and 3 1/2 disk drive, a used 10 meg hard drive and

an AT compatible keyboard.

To do it your way, and to solve problems as you go, read on... Put on

your 70 Nanosecond secret decoder ring, and let's blast off!

Step One

Buy a Magazine!  (!?!)  An indispensable tool is Computer Shopper

Magazine, published Monthly and available everywhere.  It lists the

CATALOG prices for everything you need, in all the various options.  If

you have three weeks to wait, you will get the absolute best prices on

whatever new and current you want (see Computer Shows for the

exception).  Otherwise, you will have the basis for comparison for

shopping elsewhere.  A general rule of thumb:

Catalog prices are

- 40% Lower than Discount Store Price

- 60-80% Lower than Computer Store Price

- 20% Higher than Computer Show Price

there will be exceptions, of course.

Example: 1 Meg 80 Nanosecond RAM Chips

Computer Store- $30 each

Warehouse Store- $18 each

Catalog- $9 each

Computer Show- $6 each

The above are actual prices updated regularly with this article.

Spend an entire evening reading this magazine cover to cover- it is

equivalent to an entire College Course in 6 hours.  Your head will be

stuffed with new information and insights.

A Brief Description of Your Buying Options; advantages and

disadvantages.

Computer Shows

Held around the area by different Companies- watch the local paper

Business Section and the back of Computer Shopper Magazine. Careful

buying is the watchword.  Buy all your Cables and miscellaneous parts

here.

Ask lots of questions about other pieces from information you gleam

here, from Magazines, and from Books you run across.  Knowledge is

Power.  They will mislead you to make a sale- but won't make any

patently false statements.  My purchase of a 3 1/2 Disk Drive was a

great buy until discovering from the Panasonic Technical Department that

it won't run in Motherboards made after 1985!  But the Dealer mailed me

a refund.

Catalogs

As stated before, the main drawback is the wait; and don't forget that

you have to add shipping and handling costs to the price.  Invest the

dollar to call the Advertiser's Technical Line and ask questions: is

this Hard Disk Controller MFM or RLL? Is it 8,16 or 32 bit?  (don't get

thrown by this jargon- you will learn it quickly.  It's analogous to

asking Is it Front or Rear Wheel Drive?  4 or 8 Cylinders?  Someone who

never drove a car would be just as thrown by those questions.)

I have recently had a real nightmare dealing with a Mail Order house in

Texas. We were up to 4 bad shipments before we called Computer Shopper,

who laid down the law to the turkeys. An exasperated president called

the day after we called Computer Shopper and said "What do you want?" We

told him, and we got it. Air Freight. Free. Most houses in Computer

Shopper are very reputable. Just be careful, and call the tech line and

ask lots of questions. If they have no time for you, you have no money

for them.

Discount Store

When buying a piece that may take two or three times to install right-

such as a Disk Drive- the extra money may be worth it. Questions like

Which pin is this jumper set on to make this Drive B and High Density?

is tough to answer if you are a Catalog Dealer.  Make your decision

based on how comfortable you are installing the part.  The best source

of all is the sales and repair place you now use for work.  Making

friends with your Service Rep gets you into the back room- where all the

Used but working Parts are kept (and can be bought!!) Where do you think

all the parts went when you upgraded your XT to a 286 at work?

Computer Store

BIG Companies get BIG prices having slick salesmen who frown knowingly

at your questions.  They don't want you to buy a part-they want you to

buy a System.

Your first buy:

The Case

The XT

An XT style Case  Allows for an 8088 based Motherboard or a Baby AT-

most motherboards will fit in here.

The AT Case

The "true" AT Case has become a dinosaur. It's a BIG case designed for

an AT size Motherboard. Almost every motherboard I have seen in the last

year fits into a Baby AT configuration, and the new AT size cases have

provisions for fitting a Baby AT board. You would want to pick this case

if you have saved money by purchasing a full size hard drive that will

not fit with two drives in a smaller case.

The Baby AT

(Aw...isn't it cute??) Uses a Motherboard sized to fit in an XT size

case.  My original choice.  Most AT parts (except a full size

Motherboard, full size Power Supply (XT size) and some 16 bit cards fit

in here.  Takes less room, just as fast.

Avoid "ultra slim line" and such cases. If a standard peripheral won't

fit, you are headed in a woefully wrong direction. I have recently seen

some jet black systems. They look really neat, but I haven't seen just

the case in black yet. Maybe I'll paint mine. Maybe not. What do you

care?

The Tower

A case for those of us who spend extra for a Turbo engine in our car.

Sleek, elegant, and powerful looking. It stands vertical instead of

horizontal. The Power Supply is usually bundled with the case. You can

achieve the same effect (mostly) by buying a $6 stand at a show. There

are two types of Tower cases; those that have to be taken apart like a

standard case (i.e. 6 screws in the back) and those with a removable

side panel that allows access to all peripherals with the turning of two

large screws. This panel makes the IBM PS2 80 a dream to work on.

The Baby Tower

This article just keeps getting longer. The new entry in the field is

pretty reasonable priced. I picked mine up with power supply at the last

show for $129.

Expect to pay $25-80 for the case, over $150 for a tower case with Power

Supply. Buy with confidence from a computer show or catalog.  Hard to

mess this one up.  Look for panel lights, a keylock, and a flip top

(wish I had one!).  You are inside this babe 30-40 times while building,

tweaking and adjusting, and a flip top will make your life easy.

The PC HELP pick: The Baby AT Tower with removable side.

Installation

Install the small speaker and LED's (light emitting diodes) and lock, if

any, on the case.  The speaker may get annoying.  It is loud.  Radio

Shack can help you install a little switch to turn it off and a

headphone jack for private listening of the annoying beeps.  (This is

where I used the soldering gun). You need to mount these so that the

wires don't pull out when you remove the case!

The Motherboard

Now you have a real decision to make- once you make it, you have decided

on the basic Architecture of the Machine and there is no turning back.

You now need to decide whether to buy a 8088, 80286, Baby 80286, 80386,

80386sx or i486 Motherboard.  Many Manufacturers.  Buy whichever one you

choose with the highest Mhz rating you can afford.  Mine is 12 Mhz.

Saving a few dollars by buying an XT board will limit you to a non-

reusable keyboard (unless it is switchable) and hard disk controller, a

hard to replace BIOS, and no Windows 3.0 or other newer software.

INVEST in the Motherboard.  Spend the extra $20-50 to get the best you

can afford.  Skimp on the replaceable.

The 8088 (XT) Board

Somewhat slow but can be made faster depending on options and speed up

boards.  8 bit technology.  For The Person Who Has Lots Of Time To Wait

for Spreadsheets To Recalculate.  The technology will soon be as passe

as Leaded Gasoline. Not recommended unless Price is the only

consideration.  Analogous to buying an Atari 2600 (lots of cheap

programs around!) rather than a Nintendo or a Turbo-Grafix 16 bit game

system.  Why buy something that will be expensive to speed up?  Operates

at 4.77 Mhz, with most Motherboards now being "Turbo" (8 Mhz). Most DOS

based resources are downward compatible, though, and you certainly can

build an XT for a truly economy minded system.

Consider this: The 8088 Motherboard can be had for less than $80. IF you

go this route, you need to know if it can support high density floppy

drives. If it doesn't, you will be limited to low density floppies.

Upgrading an older BIOS XT to run these drives and to support VGA

Monitors will cost you $49 from the Catalog. Plus $10 shipping,

handling, postage, etc. $80 + $49 = $129. Cost of a 286 Motherboard:

$95-$119. You may be getting the impression by now that I am trying to

talk you out of an 8088. You got it.

The 80286 Motherboard

The easiest to get cheaper, new technology pieces for.  The current

de-facto standard.  16 bit. Fast.  Accepts many 8 bit (XT type)

peripherals until you can afford rocket fuel.  8-20 Mhz. Buy as high as

you want to fly. The lowest you should go.

The 386DX and 386SX

For those of us who just have to know the last digit of Pi before

dinner. (Pi should be saved for after dinner. Sorry).  Very, very fast.

Will be the standard for the early nineties.  A forward thinking choice.

If price is less of an issue, the way to go- the price is coming down

every day.

Many people I have spoken to advise against the 386SX as being a step

backward, and that a fast 286 will beat up a stock 386SX. The other side

of the debate is that the SX has the technology on-board to keep it from

becoming passe when developers bypass 286 technology. It gets down to

speed versus being able to use tomorrow's technology. As my mentor Ken

says, "speed ain't everything." You make the call. If this machine is

going to be useful when DOS goes bye-bye, 386SX will keep you in the

race at a price currently quite a bit lower than 386. When I build again

next year, it will be 386. It's getting to the point that its hard to

not recommend building a 386 instead of a 286. Motherboards for the 386

are currently show priced at $399. This may be below $300 in a year.

This is good. The more people make the jump, the lower the price will

be. We're out of order here, but let's finish the discussion on the main

processor. The i486 is just starting to show up as a separate

motherboard. I really don't see any reason to make this plunge if you

are building to save money, unless you need it to run your power plant

or pacemaker or something.

The i486

Just coming on the market in large quantities. I don't have enough

experience with them to say whether the speed is noticeable over the

386DX.

The PC HELP pick: AmpTron 286 with 0/1 wait state, 256/1 Meg sockets, on

board Set up in BIOS.  Other Manufacturers with a good reputation

include Morse and Everex.  Pay $95-250 based on features.  Buy from the

Catalog. The latest system I helped build had made enormous strides over

just six months ago. Expanded Memory (instead of extended) was supported

right from the Motherboard. Another 4 Megabytes of Memory was supported.

Memory caching was built in. All for the same money.

Time out: A word about the BIOS

For whatever board you buy, you must be aware that next to the actual

processor, the most important consideration is the BIOS, so it deserves

it's own blurb.

The BIOS (Basic In-Out system [ooh baby])

Three chips, two for the machine and one for the keyboard. Look for BIOS

built in ($50-100 to add, AND there very well may be compatibility

problems mixing and matching), the word NEW in the Ad (you don't want

something made in 1986; the BIOS needs to be able to work with the new

small Disk Drives).  A good question to ask: Does the BIOS read High

Density 3 1/2 inch Disks?  If you don't want a high density disk now,

you will soon.  Will it take 1 Meg RAM chips? (Cheaper and leaves room

for expansion vs. 256K Ram Chips)? Some take SIMMS or SIPPS modules,

which hook 9 chips together; that's OK.  Again, this question will help

you get a board that is fairly new.  How many expansion slots?  Some

Tandys (Radio Shack) for example only use Tandy Expansion Boards (the EX

and HX).  No fun.  OK machine, but you are left out of great deals on

boards and peripherals. What is the date of the BIOS? The "main

instructions" to the CPU are handled in these (usually two) chips.

Award, AMI and Phoenix all make BIOS chips, and they upgrade them

constantly. Mr. Mail Order is all too happy to unload a Motherboard with

an old BIOS to Mr./Ms. NewBuilder. I'm not an expert on BIOS chips, but

will pass along that American Megatrends (AMI) is my BIOS of choice, and

produces (or has produced for them? I don't know which) special BIOS

made in cooperation with Chips and Technology (C&T) that come in two

flavors- EC&T which has extended BIOS settings for the true nerd and

DC&T with diagnostics routines built right in. Just don't try to get

tech support from C&T. A VERY good reason for getting new BIOS

(currently April, 1990) is that they (AMI) have a SETTABLE hard drive

table!!! Don't like the choices? Pick your own poison. If you don't know

what I'm talking about, you probably won't lose any sleep over the

difference.

Micro Channel Architecture (MCA)

And now a word from Big Blue. IBM wants yer money. They came out with

Micro Channel for two reasons: 10% to improve the computer, and 90% to

destroy YOUR ability to build your computer with inexpensive, readily

available parts. Ignore it, and maybe it will go away. Makes every card

you bought Useless (notice the capital U) by changing the socket that

fits into the motherboard. Boo Big Blue.

Installation:

Fits into the case with screws and set-offs.  Very easy to assemble.

Many Motherboards come with instructions. Mine didn't.  If not, take out

of a library or buy a book, such as How to Build an IBM Compatible and

Save a Bundle by Aubrey Pilgrim.  Handle this Baby with CARE (the

motherboard, not the book.)  LEAVE it in the plastic wrap till you are

ready to install. Put the little white plastic set-offs on the board

first. Touch the Metal Case before touching the Board. Practice

inserting the Motherboard twice before actually putting it in. Visualize

how it is going to slide in. Don't mess it up here!

Plug the Speaker leads and the Power/Turbo LED's on the Motherboard as

indicated.  If they don't work when you fire it up, reverse them. Set

the jumpers per the instructions. Pick 0 wait state if your chips are

LESS than 100 Nanoseconds (you'll read about them later). Read slowly

and carefully. It all makes sense. No, you probably don't need a Math

Co-Processor- unless you are related to Albert Einstein.

Power Supply

For an XT, somewhere around 150 watts is sufficient.  An AT, you should

spend an extra $20 to go to 200-250 Watts. The higher the wattage, the

more junk you'll be able to stuff inside.  For a Baby AT, buy an XT

Power Supply with a high rating (200 Watts). A 386 may need more

wattage.

My choice: A 200 watt XT Supply (fits a BABY AT). I chewed up two 130

Watt Supplies. Buy from a Catalog or Discount Store.  Power Supplies are

the most often repaired item.  Don't buy it used.  They go up.  Pay

$35-55.

Installation

Slips into the case via two slots at the front of the supply, then

screws to the Case.  Has two plug-ins to the Motherboard.  Make sure the

black wires are next to each other when you plug them in.  Has 5 or more

plugs to go to Drives and such. The little one goes to the 3 1/2 inch

drive. Except for that one and the ones for the motherboard itself, they

are all the same.  Make sure it comes with a Power Cord!  If not, it's

a cheap item ($3 at a show).

You will be tempted to check your installation and turn it on for a

second. If you do, you will hear a series of annoying beeps as the

Motherboard comes to life, finds no Monitor, Controller, Drive or

Keyboard and promptly bails out. You should hear a gentle whirr from the

Power Supply.  If you see sparks or smell ozone, shut it down (quick!)

and start over.

The Keyboard

May be bought refurbished from a store.  You MUST buy an Enhanced

keyboard for a 286 or higher. Old XT's used a different processor, even

though the connector still fits! The enhanced has some neat extra keys,

extra Ctrl and Alt Keys, and F11 and F12.  Some keyboards are switchable

from XT to AT.  Pay $20 (used) to $100 (fancy extras).  Look for a nice

click when you press the keys, LED's for NumLock, Caps Lock and Scroll

Lock.

Installation:

Has a round plug. Plugs into the back of the case. IBM has a little

round plug on their PS2's, so make sure it's AT compatible.

The Monitor

Ok, now we come to some pure decision making.  To keep the as built cost

down, consider giving up color.  Just for now.  Trust me.  If you want

to really plan for the future, though, and want to add $300 that will

make you happy in the long run, go for the gold and add a multi-sync or

multiscan Monitor.  Top of the Line.  Cream of the Crop.  Will run

anything from Mono to VGA, and most probably, anything coming down the

pike for 5 years.  I advise AGAINST anything in between.  If you buy

something between Mono and Multi, you will have to throw away or trade

for next to nothing to upgrade.

The choices are:

Mono:

Monographics Monitor.  Green, white or amber on a black background.

Does graphics, though!  It's hard to beat a Leading Edge Amber Monitor

at $69, available at General Computer.  Don't try to use a TV, even if

it calls itself a "Monitor".  A TV only does 40 columns across

(characters) and you need 80.

CGA

Color Graphics.  Also called RGB for Red, Green and Blue.  Shows 4

colors (figure THAT math out!).  Tandy CGA shows 16.  Nice, but a

$200-400 investment that is going passe.  Many graphics programs demand

EGA.

EGA

Enhanced Graphics.  16 colors at one time.  Was the high end standard a

few years ago.  Go higher or Mono.

VGA

Video Graphics Array.  Puts a picture of your Mother on the screen.  In

blushing color.  This is where you want to be eventually. "Real" VGA is

.31 dot pitch or LESS. VGA comes in two flavors: analog and digital. In

most electronic and audio applications, digital is better than analog.

In VGA, it's the reverse. Analog is better.   You need EGA or VGA to

fully run programs like Freelance Plus (Lotus).  You are almost there.

Only problem here is, what happens when VGA is supplanted next year?

Read on.

Super VGA

Super VGA is enhanced VGA. Has to do with the number of lines of

resolution on the screen. A monitor whose resolution is 800x600 is

considered Super VGA.

Multisync

You have arrived.  Will run anything.  Works anything.  $300 for a

perfectly good AOC or Morse to $700 (discount) for a NEC Multisync 3D

and up.  Check the Catalog.  You can find familiar names like Sony and

Toshiba here. CGA runs at a certain frequency, as does EGA and VGA.

Hence, multiple synchronization.

So Mono now, Multisync and VGA later. Read on for the exception.

A Word on VGA

VGA cards are crashing in price. Couple that with the fact that you can

get a "paper white" VGA Monitor (black and white) for $89. Hmmmm...check

it out at the Show before you go Mono. May be irresistible.

The PC HELP pick: The Monochrome VGA Monitor for starters.

Installation: Plug the Monitor AC cord into an outlet, or some plug

directly into the Power Supply of the Computer from the back of the

Machine.  Plug the Monitor Cable into the Video Board (next).

The Monitor (video) Board

For whatever monitor you buy, you have to plug a board into the computer

to run it.  A used Mono board can be had for $20 everywhere.  All those

boards that came out of old PC's and XT's and all the Manufacturer

over-runs are sitting around waiting for you.  So you can have up and

running video for $89 or so.

MONO (non-VGA)

Look for: A Hercules compatible card.  This board `interprets' graphics.

With a simple program such as SIMCGA available on bulletin boards or

from clubs, will run most CGA Graphics programs. A bit of a pain. Also

look for a parallel or serial port built in. Saves another $10-60.

Hercules is interesting; a program written for hercules has graphics

better than CGA, EGA and many VGA versions. The graphics are tight and

crisp. Centerfold Squares (Artworks, Inc. My favorite sexist software)

looks better in herc than in Super VGA! But finding herc programs is

getting more harder all the time, and you are still talking black and

white.

COLOR (and Mono VGA)

CGA, EGA and VGA cards plug in the same way.  May need software

(provided) to run, and EGA and VGA may have memory slots. Some VGA cards

are downward compatible; that is, you can run Mono, CGA, EGA or VGA with

them.  These boards fluctuate WILDLY in price.  Stores may ask $200 for

a CGA card, $300 for EGA, $400 for VGA- yet at a recent computer show,

a full function VGA card was being sold for $69.  That is why VGA is the

way to go. You must not plug a VGA monitor into a CGA board or something

like that. Make sure your card and monitor are compatible.

There is no need to buy an 8 bit VGA card if you are building an XT.

Simply place a piece of electrical tape over the second connector, and

use the 16 bit card as an 8 bit until you get a 286/386 motherboard.

There may be 16 bit VGA cards that don't work this way, but I haven't

found one.

VGA cards are rated by memory; a 256K board is less powerful and slower

to refresh the screen than a 512K board. 256K is certainly adequate for

a starter. I have recently upgraded to 512K, and I don't see much

difference.

Installation:

Plug the board into an expansion slot, advisably the farthest left.

Plug the Monitor cable into the small receptacle on the back of the

card. Use care here. Trying to plug in a VGA Analog connector into the

board blind is the leading cause of Monitor trouble: it fits just enough

to push a pin or two back up into the plug. If you are having trouble

with your monitor, inspect the connector VERY carefully. Gently pull any

pushed in pins back out (With the Monitor OFF), taking care not to

deform the pin. Some Monitors get their power from a plug that goes

directly to the Power Supply.

The Memory

THE place you will have to do some digging.  Think of Memory as a

commodity- the price fluctuates day to day, and when you ask for a

price, the dealer will likely pick up the phone and call some Chip

broker for the latest quote.  Seriously!  This is due to 1988's chip

shortage.  Prices have come way down, but, like gold, some dealers kept

the highest price.  PLEASE buy from a Catalog or a Show.  Compare

prices.  Be patient. A national software chain sells 256K chips for

$19.99. Pay $2 at a show.

Chips come in 64K, 256K and 1 Meg sizes, plus new "banks" of chips in

new motherboards called SIPPS and SIMMS. You may be able to use SIMMS,

but the price may be prohibitive for a while yet. Your Motherboard came

populated (with chips) or at 0K (most likely).  There will be a bank of

4, 6 or 8 rows of empty chip sockets.  A Memory chip looks like a small

after dinner mint with teeth.  Your Motherboard documentation (however

little they provide) will tell you that it takes one or more of these

size chips.  Use the highest you can.  It takes 9 256K chips to make

256k of memory (or 9 1 Meg ships to make 1 Meg). The ninth chip is for

parity checking and other good stuff.

Motherboards require you to fill two banks (rows) of sockets with chips

to work.  You will have to use 18 256K chips to make 512K of Memory.  If

you can use 1 meg chips, fill the same two banks with 18 chips and you

have 2056K of Memory!...and you still have empty rows you can fill

later.  1 Meg is the way to go.  Buy from a Computer Show or a Catalog.

If you call a dealer, he probably bought from a catalog, and will add

lots of dollars to that price.  Pay $2-5 each for 256K, pay $5 to $8 for

1 Meg. Some dealers sell "Pulls", i.e. chips they have pulled from other

machines. You make this call: you are taking a chance. Don't pay more

than half of a new chip's cost.

Speed

Chips vary in speed.  The LOWER the number, the faster the chip. Usual

values are 70 Nanoseconds, 80 Nanoseconds, 100 Nanoseconds and 120

Nanoseconds.  Believe it or not, many vendors charge the same price,

what ever the speed.  You can usually have one bank of 80 and one bank

of 100 or 120, but cant mix them in the same row. Lower number chips may

be TOO fast for your machine. That is why new boards are 0/1 wait state

switchable. If the chips are running too fast, you switch back to 1, a

longer wait state. Read up on it.

How much Memory?  Anything less than 512K (two banks of 256K chips) is

impractical if you are even going to run a good game or a spreadsheet.

640K- 1 Meg is the most "conventional" Memory DOS can address, and

anything above that is used for a RAM Disk (making your computer pretend

it has an extra disk drive), Print Spoolers (sends printing jobs ahead

and reducing your wait), disk caching and new stuff every day. Windows

3.0 uses as much memory as you can throw at it, and uses it well.  1024K

is usually plenty to start, makes 640K Conventional and 384K extended

(not Expanded) Memory available, but with 1 meg chips, you have to fill

2 banks, so you get 640K Conventional Memory and the rest Extended

Memory.  As more programs add uses for so called above board Memory,

there will be more uses for it. Expanded Memory (also called LIM for

Lotus, Intel, Microsoft, the consortium that approved it) is somewhat

more useful, "paging" in and out in 64K chunks to imitate regular

Memory, usable in Lotus 123 and Microsoft Windows and more programs

every day. Your Motherboard documentation will tell you whether you have

Extended or Expanded Memory on board. Some motherboards say "expanded"

and it really is extended. If it is expanded, it comes with a driver to

page in and out in 64k chunks. It uses the same chips. A few of the

newer 386's use Static RAM chips. Some static RAM chips cost $40 APIECE,

and you can't get them even if you have the bucks. Read before you buy.

386 Clone Motherboards use Extended memory for the most part, and you

turn it into expanded as needed by using an Expanded Memory Emulator,

such as QEMM.

If you have never installed chips before, take apart something old, like

a radio or answering machine that no longer works (every house in

America has a broken answering machine, I think).  Make sure it is

unplugged (of course) and find an IC chip (described above) (one that's

in a socket, not soldered in) and insert a small screwdriver under the

chip as far as it will go without force. Pull up gently. Stop. Slip the

screwdriver a little further in. Pull up gently. Stop. Insert the

screwdriver under the other end. Pull up gently. That should do it. If

you can't pull it out, there are IC extractors available very cheap (and

very expensive. I had to buy a $60 one for special job) at electronics

stores. Repeat until the chip comes out. Removing chips is a developed

art. I still break them, and the genius manufacturers couldn't be

bothered to key the chips to only go in one way. Put it back in. Repeat

this a number of times.

Go ahead and, with help if needed, install the chips and set the

Motherboard. When you go to install the chips, they are usually put in

with the notch facing the power supply.  You usually have to bend the

pins on new chips SLIGHTLY inward. Use something with a flat edge like

a plastic ruler and bend them gently, gently all at once. Make sure they

go in straight and all pins go in. The notch on the chip is usually

matched to a notch on the socket. Get help here if unsure!!! Install the

first chip in a socket that is easy to define, and the others will line

up. Inspect each chip with a flashlight when the chip is in but not

pushed down all the way. Make sure that all pins are inside the sockets,

or start over. When you are comfortable, push down firmly, but not hard

enough to bend the motherboard. Touch something Metal before handling

the chips.  Static electricity can make them instant idiots, erasing

everything they learned at the factory. The sockets may be designed to

accept both 256K and 1 Meg chips. COUNT THE PINS and match up the number

of pins on the chips and in the socket. It is incredibly easy to put the

chip in the wrong size socket. There will be a dipswitch on the

Motherboard that you will set to tell it how much Memory you are

installing, and what kind of monitor you have. The documentation will

explain the switch.

Remember, 9 chips in a row for every unit value of Memory (and I said no

Math..oh well).  9 256K chips makes 256K of Memory.

Disk and Hard Drives (I/O devices)

Before you can load a program or save a file, you need to put it

someplace semipermanent.  This is because when you exit a program or

turn the computer off, everything disappears!  Lost forever.  Gone. You

need a device or devices to save to a floppy or hard disk.  For budget

purposes, buy a new or used standard 5 1/4 inch floppy disk drive.  Try

to buy a half height drive, which only takes half the slot in the case,

so that you can add a second 3 1/2 drive later.  The disks are cheap and

will store 360K of information (1K is about 1 page of printed text).

2/3rds of programs you buy will be on 5 1/4 disks.  Pay $20-40 used,

$30-60 new. 3 1/2 drives make your life easy. If you can afford it, buy

a 3 1/2 at the same time you buy the 5 1/4. The 5 1/4 is recommended

because most programs you buy come with these disks as the default, and

if you have to boot from this disk (in many games), you need this size

drive and you need it to be "A".

NOTE: If you ever have to call the B drive A for a program, go to a DOS

disk or subdirectory and type ASSIGN a=b,b=a. That will temporarily

reverse the drive assignments. To get back, go back to DOS and type

ASSIGN by itself.

Installation.  You need a board.  Read the rest of this article before

deciding.

You need a hard/floppy or floppy controller.  You need it to be MFM or

RLL if you will soon buy a Hard Drive. There are other exotic hard drive

types explained later, but, dollar for dollar, MFM still seems to be the

best value, and most compatible. If you are building an AT (286), you

will want 16 bit (two card edges on the bottom) if you want a Hard

Drive.

The Controller

For floppy disks only: you can get a half-card floppy controller cheap.

Plug it into an 8 bit slot near the power supply.  You need a cable to

go from the controller to the back of the drive. The plug will be marked

1 on one end and 36 on the other.  1 usually goes on top. There should

also be a different color wire on one end of the "strap" of wires. This

color designates wire one. If the disk drive lights and doesn't go out

when you fire up, you have it backwards.  The other end of the cable

should have two card edge connectors, marked with numbers as described

above.  There is also usually a slot in the female portion keeping you

from putting it on backwards.  One card edge connector may have a twist

in the cable near the connection.  This and the fact that it is the end

of the cable indicates the Drive A connector.  Plug it into the back of

the drive, which you have slid into the slot in the case.  Plug one plug

from the power supply into the drive.  If you have bought a second

drive, the other card edge connector hooks to that drive to make it

drive B. You MUST follow the manual for the drive or call the

Manufacturer to set the little jumpers on the drive near the back to get

the drive configured correctly. To me, this is the most frustrating part

of putting a system together. Most manufacturers use jumpers which are

tiny black sockets that connect two pins together. They are near

impossible to remove with or without tweezers, and sometimes you end up

having to move these jumpers 10 times or more to get the configuration

right. Hey manufacturers, use dipswitches! With a twisted floppy control

cable, set both drives to the second position, which may be DS1 or DS2,

depending on whether the manufacturer starts counting at 0 or 1. In any

event, it's the second drive position.

For hard drives, you need a separate controller, or a Hard/Floppy drive

combination controller. In most cases, you will be using the latter.

Follow the documentation.

Types of Floppy Drives

Besides the old standby 360K 5 1/4, there is a high density 5 1/4 that

stores 1.2M of data.  This drive is problematical.  You will not be able

to write to a low density diskette and use it on another computer with

a low density drive.  There are also 3 1/2 inch drives.  The disks it

uses are hard, less likely to go bad, and fit in your shirt pocket

without a sleeve.  Wonderful.  Buy one as a second drive.  Stores 720K.

High density 3 1/2 drives store 1.44M, and aren't as problematical as

high density 5 1/4's and surprisingly, are only about $10 more than low

density!  Buy the high Density.  Pay $30 (used) to $75.  Installation

note: I had my 3 1/2 high density drive set to Read Media, which meant

that the drive decided which type of disk was in the drive.  A friend

gave me a program on a high density diskette.  The machine wouldn't read

it.  To make a long story short, she had formatted the high density

diskette to low density, and the drive to trying to read high.  I

changed the jumper to read the disk type from the machine instead of the

Toshiba drive. Problem solved.  If your BIOS routine sets disk types in

the Setup program, do it this way.

ANOTHER NOTE: This will pay for reading this article many times over. If

your BIOS is set in the set up routine to read high density diskettes,

and the drive is high density, setting the jumpers right on most models

will let low density diskettes format to high density! There. I've just

saved you $3 a diskette for the rest of your life. Send $10 to

Greenpeace.

Caution: Some computer manuals claim that you should never do this. They

say that the low density disks are not made to be formatted high, and

you will lose data. I have NEVER had this happen, so you make the call.

If the only copy of your will is on a low density disk formatted to

high, back it up somewhere. Personally, I could write my will on the

label of a 3 1/2.

Hard Disks

Ok, ready for some jargon?  There are MFM drives (usually old) RLL, SCSI

(pronounced scuzzy for some warped reason), IDE and ESCI drives.  Buy

whatever you get the best deal on, and fills your purposes (if you can

afford ESCI, you probably aren't interested in building) but your

controller MUST be compatible with that type. buy a 16 Bit controller

card. You can get 10,20,30,40,80 and up Megabytes of memory.  Read up on

it.  30M is about the best compromise for the limited-income builder.

Buy a compatible Hard/Floppy controller (MFM or RLL [higher capacity by

increasing the sectors]) to match the drive and 8 bit for XT or 16 bit

for AT) to control all the drives.  Buy a half height if possible to

save room for a second hard drive.  I installed a 10 meg full height

(great buy!) and filled it in two days.  Word Perfect took 2M.  Symphony

took 1.5M.  Windows 3.0 needs 3-6 million and is worth every byte. And

on and on.  Pay $50 (used) to $600 for a hard drive. Pay about $225 for

a 20 Meg. Get a book on formatting. Get the shareware programs IAU and

HDDIAG from a bulletin board or a Computer Show. Ready for a crap shoot?

Show up at a show AT OPENING TIME. Someone will invariably be there

selling hard drives pulled from old machines for $1 a megabyte. For

thirty dollars you can take home a 30M drive that has a 50-50 chance of

working. Like I say, you make the call. I bought a 42MB voice coil (as

opposed to stepper motor) drive for $40; it only needed re-formatting.

Then again, chances were just as good that this drive was used as an

anchor for a crab pot for six months. Again, it's a crap shoot.

Hard drives come in full size, half size (both 5 1/4) and 3 1/2 size. BE

SURE that the case you buy will take a full size hard drive and two

floppies before you buy a full size drive.

Drives are also split into Stepper Motor and Voice Coil. Be aware the

stepper motor will give you great service, but cannot be moved without

"parking" the drive. A stepper motor hard drive is just like a

turntable; if you shake the drive, the needle will go skittering across

the surface of the drive platter, destroying data as it goes. Most hard

drive repairs are reformatting as a result of the user banging the

machine around, or, (GASP!) turning the machine over and shaking it to

get out a loose screw. Voice Coils can be safely moved, and are more

expensive. Ask the seller which the drive is.

High Tech Hard Drives

SCSI works by putting the controller on the Hard Drive, and the floppies

work off of that controller. IDE is an animal unto itself. IDE also puts

the controller on the drive, and then plugs into a card (not a

controller?) that also has the parallel, serial, game port and floppy

controller on the same card. I just installed one in my system. I can

see NO speed improvement over my Adaptec MFM controller.

Installation

There is a big cable that goes from the Hard Drive to the controller and

a small one.  Match pin one to pin one all the way around.  Slide the

Hard Drive into the case, connect these cables, and plug in the power

supply lead. For two drives, get a book and follow the instructions.

It's not that hard, but you have to deal with twisted cables and

terminating resistors and such. The terminating resistor goes on the

last physical device.

The hard drive must be set up in your BIOS program, then low-level

formatted and high level formatted. Pick up a book at the library,

borrow one or ask a friend. The procedure is not difficult. The two

aforementioned share-ware programs IAU and HDDIAG makes the process easy

and configures the drive to it's optimum performance.

In 286 and 386 machines, you must pick an entry from your Set-up

program, which comes up when you start the computer. It is accessed by

a particular keystroke, referenced on the screen before the machine

actually boots DOS. The message will say something like "Press DELETE to

enter Set-up". Set-up writes your component information into the CMOS.

CMOS stands for something or other which I always forget.

Shareware programs are available to help you pin down which "number" of

a standard set of numbers your hard drive type is. An old IBM 10 meg

drive is #1. An old Seagate 412 is #23. The BIOS will ask you for this

number. If you bought from Crazy Johns Unguaranteed Hard Disk Bargain

Nearly New Shop, you may have to do some digging to find the number.

Disk Manager, which comes with Seagate Hard Drives, will interrogate the

drive for you, but is a real pain in the butt when the drive isn't

Seagate. Here's a real kluge. My hard drive didn't fit my table, so I

hooked the hard drive up to a machine with a settable drive table,

formatted it, then saved the settings to the disk with Disk Manager. If

the drive ever crashes, I'll probably have to repeat this process or

update my BIOS. You need to know the number of cylinders and number of

heads, at a minimum. The number of sectors for MFM is usually 17

sectors, RLL 26 or 27. Call the local computer club to track down a list

of hard drive specs. You match the number of cylinders and number of

heads in the drive table in your set-up program WITHOUT GOING OVER. Just

like the Price is Right (hi Bert!). If your drive has 8 heads, you can

pick 5,6,7 or 8, but not 9.

Skip this paragraph unless you end up with an IDE drive. I just learned,

by force, how to set up one of these bears. The IDE drive table entry is

determined by MULTIPLYING the number of heads x the number of sectors x

the number of cylinders, and matching that to the drive table entry that

most closely equals the number of heads x cylinders x sectors. Sheesh.

The advantage is, you are not limited to the closest without going over

rule. The one thing you MUST NOT DO is low level format an IDE drive!!!

They are low level formatted at the factory. If you do a low level

format, it becomes an MFM drive. This exciting information was passed on

to be by the Seagate technical department AFTER I had low level

formatted three times trying to ge the bear to work. There was NOTHING

in my documentation telling me not to do a low-level. I had to get

another drive and start over to get the full performance from the drive.

FORMATTING

Formatting the hard disk is involved but not difficult. HDDIAG and

AMIDIAG (AMI, the BIOS people) are both free or shareware programs

available at Computer Shows and through magazines for a couple of bucks

each (Registration will cost more. These folks deserve their money.)

They will walk you through the low level formatting, which prepares the

disk for your system. Seagate and some other manufacturers come bundled

with Disk Manager, which is a great utility, but only works with that

brand hard drive. Next you run your DOS program FDISK, in which you set

up the DOS partition. If you are running DOS 3.3 or less (hardly anyone

likes DOS 4.01) you are limited to 32 Megabytes per partition. Thus, a

40 Meg harddrive must be split up into more than one partition (C and D

[and maybe E]). A drive runs faster if you minimize the C partition and

maximize the others. The D and above partitions run much faster. No big

deal to run FDISK. Finally, you high level format the disk, i.e. format

C:/s which makes the hard disk ready for use and bootable and format d:

which prepares the D drive. Take this whole process slow and easy. Block

out quiet time to do it.

PORTS

You may not be ready for this yet, but here goes. There are Ports for

adding peripherals to your machine, just like you add a CD or turntable

to your stereo. A mouse usually needs a serial port or works off of it's

own card, and a printer a Centronics Parallel port. The following will

NOT a full diatribe on ports, but a starting point.

PARALLEL

This one is easy. Plug the 25 pin end into an I/O card you have bought

for the computer. Plug the weird looking end into the printer. This port

is usually called LPT1.

SERIAL

Also known as RS-232. Usually a 25 pin connection. A little more

complicated, but faster and more versatile than parallel. Don't get

scared by all the technical jargon that comes with serial port devices.

I've rewired one ONCE for a special application, and I install about 400

of them a MONTH. Serial ports most often are for Mice, scanners,

plotters and external modems and such. The serial port is the place most

computer hacker wanna-bes have trouble. They represent 80% of my trouble

calls. The first and best advice is, KEEP IT SIMPLE. DON'T mess with an

IRQ that works, and NEVER, NEVER change the memory address of a COMM

port. Okay, okay, I'll back up. There are COMM ports for serial devices.

COMM 1,2,3 and 4. You use them in order as you need them. Usually the

Modem (Modular/DEModulator), which allows you to call Compuserve and

give them all your money is COMM 1 and the mouse is COMM 2. COMM 1 is,

by default IRQ4 (Interrupt ReQuest 4) and COMM 2 IRQ3. Each has a memory

address. Here's the dangerous part: you are advised, in the worst pidgin

english in the manual to mess with flipping IRQ and memory addresses and

COMM port assignments around. Please don't!!!!! Leave it alone. Keep it

simple. Messing with them is akin to switching spark plug wires in your

car. All you need to know unless you futz with them is:

LPT1:  IRQ7

COMM1: IRQ4

COMM2: IRQ3

LPT2:  IRQ5

COMM3 and COMM4 work off IRQ 4 and 3 respectively, but at different

Memory addresses.

Don't call me if you mess with IRQ's. It gives me a headache. If you

futz with the memory address, I won't even talk to you at the Christmas

Party.

To summarize, put in the I/O board, hook up your printer (I like the

Panasonic KXP 1124), call it LPT1, hook up your preferably internal

modem as COMM1, IRQ4 and your mouse as COMM2, IRQ3 then leave it alone.

You have been warned.

SUMMARY

You have installed the Motherboard and Power Supply, hooked up the

speaker and LED's, inserted the Memory chips, set the Motherboard

dipswtich(es), put in the Monitor Card and plugged in the Monitor,

plugged in the keyboard, inserted the Hard/Floppy Controller, slid in

and connected the drive(s), and plugged the whole shebang into the wall.

You have left the I/O card and any optional devices such as mice and

printers out of the picture until the basic machine is working well.

Now turn it on.

If everything is hooked correctly, the Memory will count off as it is

checked, and you will be sent (hopefully) to a set up program in BIOS to

describe and save the configuration of your system.  You now need to add

MS-DOS from a disk to "Boot" the computer, and you are up.  The moment

you see your first A> prompt you are in business.  Open a bottle of

champagne, pour one for me, and accept my hearty congratulations.

MY SYSTEM

11/89  Current

Piece                       New or Used Source         Price

=======================================================================

Case- Baby AT                   New     Catalog          $25.00  $25.00

Motherboard-AT286 Amptron       New     Friendly Rep    $199.00 $109.00

Monitor-Leading Edge            New     Gen. Comp.       $69.00  $69.00

Mono Card w/ parallel port      Used    Friendly Rep     $20.00  $20.00

Power Supply 200 Watt           New     Disc. Store      $69.00  $46.00

Teac 5 1/2 Drive                Used    Show             $40.00  $40.00

Toshiba 3 1/2 HD Drive          New     Disc. Store      $89.00  $79.00

IBM MFM 10 Meg Hard Drive       Used    Show             $20.00  $20.00

Adaptec Hard/Floppy Controller  Used    Friendly Rep     $69.00  $69.00

Memory- 18 1M 80NS Chips        New     Show            $162.00 $108.00

Serial Port                     Used    Show             $10.00  $10.00

Modem 1200 bps (optional)       Used    Show             $20.00  $20.00

Switch and headphone jack (opt) New     Radio Shack       $4.00   $4.00

========= ======

TOTAL                                                   $796.00 $619.00

THE COMPUTER SHOW SHOPPING GUIDE       DJ Elliott             DID I

I WANNA  VENDOR                      GET A

COMPONENT  STYLE         PAY      ________ ________ ________  RECEIPT?

======================================================================

THE NECESSARIES

======================================================================

CASE                  XT   ______   ______   ______   ______   ______

AT   ______   ______   ______   ______   ______

BABY AT   ______   ______   ______   ______   ______

TOWER   ______   ______   ______   ______   ______

MOTHERBOARD         8088   ______   ______   ______   ______   ______

80286   ______   ______   ______   ______   ______

BABY 286   ______   ______   ______   ______   ______

386SX   ______   ______   ______   ______   ______

386DX   ______   ______   ______   ______   ______

POWER SUP       ____WATT   ______   ______   ______   ______   ______

FLOPPY DR.     5 1/4 LOW   ______   ______   ______   ______   ______

5 1/4 HI   ______   ______   ______   ______   ______

3 1/2 LOW   ______   ______   ______   ______   ______

3 1/2 HIGH   ______   ______   ______   ______   ______

HARD DRIVE       ___ MEG   ______   ______   ______   ______   ______

___ MEG   ______   ______   ______   ______   ______

___ MEG   ______   ______   ______   ______   ______

DRIVE        HD/FL COMBO   ______   ______   ______   ______   ______

CONTROLLER  MFM 8/16 BIT   ______   ______   ______   ______   ______

RLL 8/16 BIT   ______   ______   ______   ______   ______

OTHER 8/16 BIT   ______   ______   ______   ______   ______

CABLES   ______   ______   ______   ______   ______

FLOPPY 8/16 BIT  ______   ______   ______   ______   ______

CABLE   ______   ______   ______   ______   ______

MONITOR             MONO   ______   ______   ______   ______   ______

CGA   ______   ______   ______   ______   ______

EGA   ______   ______   ______   ______   ______

VGA   ______   ______   ______   ______   ______

VGA MONO   ______   ______   ______   ______   ______

MULTISCAN, MULTISYNC   ______   ______   ______   ______   ______

MONITOR        HERC MONO   ______   ______   ______   ______   ______

CONTROLLER           CGA   ______   ______   ______   ______   ______

EGA 8/16 BIT   ______   ______   ______   ______   ______

VGA 8/16 BIT   ______   ______   ______   ______   ______

CABLE   ______   ______   ______   ______   ______

MEMORY         256K CHIP   ______   ______   ______   ______   ______

lower         1 MEG CHIP   ______   ______   ______   ______   ______

nanoseconds OTHER CONFIG   ______   ______   ______   ______   ______

better

I/O BOARD     __PARALLEL   ______   ______   ______   ______   ______

__SERIAL   ______   ______   ______   ______   ______

__GAME   ______   ______   ______   ______   ______

KEYBOARD                   ______   ______   ______   ______   ______

======================================================================

THE GOODIES

======================================================================

MODEM      _____BPS  INTE  ______   ______   ______   ______   ______

_____BPS  EXTE  ______   ______   ______   ______   ______

_____BPS  INTE  ______   ______   ______   ______   ______

_____BPS  EXTE  ______   ______   ______   ______   ______

MOUSE                BUS   ______   ______   ______   ______   ______

SERIAL   ______   ______   ______   ______   ______

PRINTER      DAISY WHEEL   ______   ______   ______   ______   ______

DOT MATRIX 9P   ______   ______   ______   ______   ______

DOT MATRIX 24   ______   ______   ______   ______   ______

LASER   ______   ______   ______   ______   ______

CABLE   ______   ______   ______   ______   ______

FAX BOARD                  ______   ______   ______   ______   ______

SCANNER                    ______   ______   ______   ______   ______

BLANK DISKS______________  ______   ______   ______   ______   ______

SOFTWARE   ______________  ______   ______   ______   ______   ______

SOFTWARE   ______________  ______   ______   ______   ______   ______

SOFTWARE   ______________  ______   ______   ______   ______   ______

SOFTWARE   ______________  ______   ______   ______   ______   ______

SOFTWARE   ______________  ______   ______   ______   ______   ______

OTHER      ______________  ______   ______   ______   ______   ______

STUFF      ______________  ______   ______   ______   ______   ______

Get a receipt. Get a phone number and address. Get a guarantee.

The unaltered document is 9277 words.

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X

Another file downloaded from:                               NIRVANAnet(tm)

& the Temple of the Screaming Electron   Jeff Hunter          510-935-5845

Rat Head                                 Ratsnatcher          510-524-3649

Burn This Flag                           Zardoz               408-363-9766

realitycheck                             Poindexter Fortran   415-567-7043

Lies Unlimited                           Mick Freen           415-583-4102

Specializing in conversations, obscure information, high explosives,

arcane knowledge, political extremism, diversive sexuality,

insane speculation, and wild rumours. ALL-TEXT BBS SYSTEMS.

Full access for first-time callers.  We don't want to know who you are,

where you live, or what your phone number is. We are not Big Brother.

"Raw Data for Raw Nerves"

X-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-X